Vs2

i2

R2

iL

i5

i4

i1

R5

R4

R1

A

C

D

B

R3

i3

Vs1

RL

EXPERIMENT 2a. Report form.

Fig.1.Circuit under test

Table 1. Circuit elements :

| R1 k Ω | R2 k Ω | R3 k Ω | R4 k Ω | R5 k Ω | R6(RL) k Ω | VS1 V | VS2 V |
| --- | --- | --- | --- | --- | --- | --- | --- |

TASK 1. Open circuit voltage measurement.

Insert SPICE circuit enabling us to measure open circuit voltage (remove load resistor. Place this voltage in Table 2 (complete the table with laboratory measurement and homework calculations)

Table 2.

|  | Lab. Meas. | Home Cal. | Spice Simul. |
| --- | --- | --- | --- |
| Veq (VOC ) V |  |  |  |

TASK 2. Short-circuit current measurement.

Insert SPICE circuit simulating short-circuit current is measurement. Place this result in Table 3 (add laboratory measurement and homework calculation).

HINT: Model the short-circuit branch by  resistor having small value or voltage.

Table 3.

|  | Lab. Meas. | Home Cal. | Spice Simul. |
| --- | --- | --- | --- |
| iS mA |  |  |  |

l

TASK 3. Equivalent resistance measurement.

Insert SPICE circuit  Req measurement simulation. Place this result in Table 4 (add laboratory measurement, homework calculation and result following from veq and is simulations), attach calculations!

HINT: SPICE is not equipped with ohmmeter – use standard definition of resistance.

Table 4.

|  | Lab. Meas. | Home Cal. | veq/is from simulations | Req from direct SPICE simulation |
| --- | --- | --- | --- | --- |
| Req k Ω |  |  |  |  |

TASK 4. Current iL measurement.

Insert SPICE simulation of Thevenin’s and Norton’s equivalent circuits used to calculate load resistance current iL. Fill Table 5 .

Table 5.

|  | Lab. Meas. | Home Cal. | From Thevenin’s circuit | From Norton’s circuit |
| --- | --- | --- | --- | --- |
| iL mA |  |  |  |  |

TASK 5. Branch currents calculations.

Simulate all circuits (in SPICE program)  necessary to confirm superposition theorem (for circuit presented in Fig.1).

Complete Table 6.

Copy appropriate  schemes from the simulator containing current values with reference  directions.

TABLE 6

| Measured → currents | i1 | i2 | i3 | i4 | i5 | iL |
| --- | --- | --- | --- | --- | --- | --- |
| Acting sources ↓ | mA | mA | mA | mA | mA | mA |
| Vs1 |  |  |  |  |  |  |
| Vs2 |  |  |  |  |  |  |
| Vs1 and Vs2 |  |  |  |  |  |  |
| Algebraic sum of responses |  |  |  |  |  |  |