Problem 1:   Plate capacitor.

Fig.1. Plate capacitor.

Data:d=10mm, a=5cm, b=10cm ,V(0)=0, V(d)=12V

Tasks:

- Propose the FEMM planar model of plate capacitor, observe voltage (V), flux density (D) and field intensity (E) between the plates.

- Plot V(x), D(x) and E(x).

- Calculate the capacitance C using FEMM tools (energy stored). Complete the Table 1.

Consider two cases: εr1=1, εr2=5.4

Table 1

|  | Method |  |  |
| --- | --- | --- | --- |
|  | Analytical | FEMM | Error |
| C [F] for r=1 |  |  |  |
| C [F] for r=5.4 |  |  |  |

General formulas for capacitance:

Problem 2:   Double-layer plate capacitor.

Fig.2 Double- layer capacitor

Data:

d=10mm, a=5cm, b=10cm ,V(0)=0, V(d)=12Vd1=5mm, εr1=3, εr1=5,4.

Tasks:

- Propose the FEMM planar model of plate capacitor containing two layers of dielectric, observe voltage (V), flux density (D) and field intensity (E) between the plates.

- Plot V(x), D(x) and E(x).

- Calculate the capacitance C using FEMM tools (energy stored) and compare with the result obtained from analytical formula. Complete the Table 2.

Table 2

|  | Method |  |  |
| --- | --- | --- | --- |
|  | Analytical | FEMM | Error |
| C [F] f |  |  |  |

General formulas for equivalent capacitance:

Problem 3:   Cylindrical capacitor.

Fig3. Cylindrical capacitor.

Data:

r1=50mm, r2=60mm, h=10cm ,V(r1)=0, V(r2)=12VCase 1:εr1=1, Case2:εr1=5,4.

Tasks:

- Propose the FEMM planar model of cylindrical capacitor, observe voltage (V), flux density (D) and field intensity (E) between the plates.

- Plot V(x), D(x) and E(x).

- Calculate the capacitance C using FEMM tools (energy stored). Complete the Table 3.

Table 3

|  | Method |  |  |
| --- | --- | --- | --- |
|  | Analytical | FEMM | Error |
| C [F] for r=1 |  |  |  |
| C [F] for r=5.4 |  |  |  |

General formulas for cylindrical capacitor :

Problem 4:   Cylindrical double-layer capacitor.

Fig3. Double-layer cylindrical capacitor.

Data:

r1=50mm, r2=60mm, h=10cm ,V(r1)=0, V(r3)=12Vεr1=3, εr1=5,4.

Tasks:

- Propose the FEMM planar model of cylindrical double-layer capacitor, observe voltage (V), flux density (D) and field intensity (E) between the plates.

- Plot V(x), D(x) and E(x).

- Calculate the capacitance C using FEMM tools (energy stored). Complete the Table 4.

Table 4

|  | Method |  |  |
| --- | --- | --- | --- |
|  | Analytical | FEMM | Error |
| C [F] |  |  |  |

General formulas for cylindrical double-layer capacitor :

Problem 5:   Two-wires line with cylindrical conductors

Let us consider, two-wires line:

d



Wire diameter =75 mm,

Distance between the wires d=200 mm.

The line is in the air with =0 .

Tasks:

- Propose the FEMM planar model of this line, observe voltage (V), flux density (D) and field intensity (E) around the wires.

- Calculate the capacitance per unit length. Complete the Table 1

Remarks:

- The analytical formula for C is ( = 0 = 8,8541878.10-12 F/m,  l=1 m,  d=0,2 m,  R=0,0375 m):

C=π ε llnd2R+d2R2−1

- FEMM 4.2 calculations: energy stored in the system (area) is needed.

Table 1

|  | Method |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Analytical | FEMM | Error | FEMM/Kelvin | Error |
| C [F] |  |  |  |  |  |